# Predicting Salinity Intrusion in the Vietnamese Mekong Delta

A dissertation in partial fulfilment of the requirements for the degree of Bachelor of engineering in computer science with a year in industry in Queens university Belfast by Christopher Mackle

Declaration cover sheet

Table of contents

# Acknowledgements

I would like to acknowledge my supervisor Dr 
Thai Son Mai for his guidance and advice surrounding the subject area. And I would like to thank my family for continued support throughout.

# Abstract

## Chapter 1 - Introduction

# Introduction

The Vietnamese Mekong delta is the countries agricultural power house, “contributing 90% of Vietnamese rice exports, 70% of its fruit and 60 % of its seafiid” [https://en.mae.gov.vn/mekong-delta-from-rice-bowl-to-sustainable-green-region-8832.htm]. So disruptions of any sort can have a massive impact to the countries economy and the well being of the citizens. For example a disruption of as little as 0.5%, without it being easily replaced for the rice alone would mean a loss of approximately $25.9million based on its revenue from 2024 where it generated “5.75 billion USD in revenue” [https://en.nhandan.vn/expanding-markets-and-maintaining-growth-in-rice-exports-post144584.html]. Then there comes one big problem every year dry season salinity intrusion, is the “process where saltwater moves into freshwater areas” [https://pollution.sustainability-directory.com/term/salinity-intrusion/]. Where saltwater moves in through the river networks, which intern pushes the salinity levels to damaging thresholds for irrigation and most importantly freshwater supply (used in agriculture). During the 2019 to 2020 dry season the UN assessments reported salinity arriving 10 to 20 days earlier than previous years, up to 110km inland in some locations, putting 658558 people at risk from drought and fresh water shortages, producing large economic losses (in 2016 there was a loss of 5.5trillion USD)[ https://pmc.ncbi.nlm.nih.gov/articles/PMC8800995/#:~:text=In%20the%20dry%20season%20of,of%20vegetables%20in%20Ben%20Tre].

Therefore the goal of my project is to build a local reproducible workflow to predict short horizon(1 to  24 hours) salinity and provide a light weight viewer for researchers. Recent dry seasons have brought earlier stronger intrusions, so short notice guidance matters and can make a real impact. The scope for the project is offline only with cleaned per station datasets, benchmarked baselines against a gate index made using physics on key stations and ship a tidy viewer for a single and 2 station side by side inspection. Add in models chose and why at a high level.

# Problem and motivation

Aim

The aim of my project is to develop and evaluate a simple reproducible salinity forecasting pipeline for selected stations and create a simple viewer to help researchers analyse data.

Objectives

- Build a robust data pipeline that will clean and merge the salinity, water level and rainfall data for selected stations.

- Implement and run four models on the same data and splits, the models being Persistence, Ridge , LSTM baseline and LSTM with gate.

- Evaluate the models performance with the same metrics across all 4, across stations and forecast horizons.

- Quantify whether or not the gate index feature improves LSTM performance.

- Produce a simple viewer for researchers.

Research questions

- How much does LSTM, LSTM with gate and ridge beat persistence for salinity forecasting?

- Does adding the gate feature improve the LSTM accuracy and is it consistent across 4  stations?

- Is a simpler model comparable or better than LSTM under the same conditions?

- Which model offers the best overall performance with complexity and or accuracy trade offs in mind for a practical deployment?

# Scope and assumptions

Scope

Assumptions

# Background

Mekong river

“The Mekong Delta is a vast spread of fertile riverbeds, islets and mangrove swamps at the end of the Mekong River, where Vietnams coastline meets the sea. It spans about 65,000sq km(25100sq miles) and is home to more than 20 percent of Vietnams population”[ https://www.aljazeera.com/features/2020/4/22/the-great-salt-drought-desiccating-vietnams-mekong-delta].

What is salinity intrusion?

Salinity intrusion also known as “saltwater intrusion refers to the inland movement of saline water – from the ocean or sea – into freshwater aquifers” [https://www.bbc.co.uk/future/article/20260216-the-unseen-force-turning-water-to-salt]. For my case the seawater from the south china sea and the gulf of Thailand feed encroach into the river system.

How does it effect farming?

The encroaching saltwater contaminates the groundwater and or the freshwater sources. When the saltwater does this it makes it “much harder for the plant roots to extract water from the soil”[ https://edepot.wur.nl/453721.]. As well as this “Sodium takes the place of potassium in the plant, preventing various proteins from working properly”[ https://edepot.wur.nl/453721l.]. These then lead to subsequent lower yields.

Prevention methods

Along the delta they have a couple prevention/ mitigation strategies such as “ prevention sluices and conducted public awareness campaigns to promote water storage and efficient utilisation” [https://vietnamnews.vn/society/1716057/proactive-measures-mitigate-saltwater-intrusion-damage-in-mekong-delta.html]. Sulice gates are the main form of protection. “Sulice gate is a hydraulic structure used to control the flow of water in open channels”[ https://drainboss.co.uk/glossary/sluice-gate/].

Forecasting

This is where forecasting comes in, if we are able to tell when the saltwater is encroaching before it happens we can give early warning and close the gates and or turn off the water pumps in the fields so its not spread on the crops.

Related Methods

This forecasting is done in a couple of ways, I found an article stating that “LSTM and XGB Models have the best prediction”. Where the study tested these against 4 other models “RNN, SVR, GB and ANN”. [https://link.springer.com/article/10.1007/s44163-025-00336-3] . For similar fields like solar radiation simple persistence and climatology is used as a baseline [https://www.sciencedirect.com/science/article/abs/pii/S0960148122005262].

Evaluation approaches

The standard approach for evaluation is to use RMSE and MAE, to measure how far off my predictions are, using the same units as the salinity levels.[ AI module?]

Add pitfalls?

# Data and Problem definition

Study area, sensors and variables

This study is focusing on salinity forecasting at a couple stations. The data is taken from the stations situated along the Mekong delta. The source data contains salinity observations as well as water level and discharge.

Data quality and missingness handling

The raw data files are cleaned and merged per station files. They are resampled into standard time frame (hourly). For short gaps (3 hours max) interpolation is used to fill them, to make the data set more useful. As well as this there is also clipping removing data that is extreme or likely error. Also the removal of rows without salinity. This is all done through a reuseable script.

Forecasting task

I am trying to predict the salanity value for that station, using other sensors and driver data. The main set up is for 1 hour a head( horizon = 1). I made it in a pipeline so that the horizons are parameters that can be changed and set to 3, 6 or 24.

Training validation and test splits

Data splitting is done chronologically because of  the seasonal problem we have. For each sequence samples are partitioned as 60/20/20. No random shuffling is used across split boundaries.

Metrics

Model performance is reported using the following Mean absolute error, Root mean square and coefficient of determination.

# System Requirements and Specification

System overview

The purpose is a local forecasting system for the Mekong salinity. It cleans and merges data, trains 4 forecasting models for selected stations and provides an interactive viewer for inspection and comparison of data.

It will be made up of these 5 components data preprocessing script, a pipeline script, 4 models (LSTM, ridge, perstiance and LSTM with gate proxy), reporting outputs and a data viewer.

For the inputs and outputs, the inputs will be from CSVs(salanity, discharge, water level and rainfall) and optionally metadata (renaming CSV). For the out puts there will be per station cleaned data sets as well as metrics csv, prediction CSVs, and viewer exported csv.

Use cases

This system will be used by researchers and government officials in the country of Vietnam. To help them make better informed desicision for there action plan going forward for the benefit of the overall country.

Use Case 1 – Data Preparation

The researcher may want to clean and merge data to produce hourly quality checked per station datasets.

Use Case 2 – Training and evaluate models

The researcher want to use that cleaned data to generate metrics and predictions using, ridge persistence, LSTM and LSTM gate models for 4 stations to compare the outcomes.

Use Case 3 – Compare model performance

The researcher may want to compare the models performance using MAE, RMSE and Rsquared outputs across stations to select the best model

Use Case 4 – Inspect the data visually

The researcher may want to load CSV or folder data in to filter by station to check trends and anomalies in the data.

Use Case 5 – Compare stations

The researcher might want to view these trends side by side to compare trends side by side.

Use case 6 – Export data from viewer

The Researcher once they have found an anomaly or a trend may want to export that to show someone else where.

Requirements functional and non-functional

Functional

| Requirement | Completion |
| --- | --- |
| The system will take in the CSV data from the station in the local folders. |  |
| The preprocessing will resample all series to a common hourly timestamp and apply aggregation. |  |
| The preprocessing stage will do some range validation, checking and getting rid of any non normal values like negative salinity values and replace them with Nan. |  |
| The preprocessing will interpolate short gaps (3 hours) in the driver variables. |  |
| The system will make one cleaned, merged per station dataset. |  |
| There should be an easy pipeline shall run in sequence cleaning, merge, feature selection then model training and validation. |  |
| The pipeline will train and evaluate all of the models, Persistance, Ridge, LSTM baseline and LSTM with date feature. |  |
| The pipeline should calculate the gate feature from rolling tidal coupling and rainfall damping. |  |
| The forecasting setup will support configurable lookback and horizons. Maybe non functional |  |
| The models will be trained on chronological train/validation/test splitting also it has to be time based. |  |
| The system shall return the per model predictions and summary metrics. |  |
| There should be a viewer that shall support interactive filtering and graphs. |  |
| There should be a compare section allowing you to compare them side by side |  |

Non-functional

| Requirement | Completion |
| --- | --- |
| Runs should be reproducible through fixed seeds. |  |
| The pipeline should have validation and fail when missing columns not reported. |  |
| The viewer should load in the data set through a live server |  |
| The viewer should have the option to add meta data(change the names used in the data set through a csv all at once for convievience) |  |
| Scripts should be modular for maintainability. |  |
| The viewer should have a landing screen showing you how to use it. |  |
| You should be able to upload you own csvs to the data viewer as a file or a zip. |  |
| You should be able to export that data created. |  |

Architecture and data flow diagrams

Data formats / interfaces

Viewer UI structure(screenshots and interactions)

Reproducibility details (configs, seeds, environments)

# Modelling Methodology

Baselines: persistence

Ridge regression( setup(features regularisation ,tuning)

LSTM setup( architecture, loss, optimser, earlystopping)

Training procedure and hyperparameter search

Model selection Criteria

# Results

Main tables/plots per horizon and station

Comparison: perstitance vs ridge vs LSTM vs LSTM with gate thing

Error analysis plots( residuals, scatter calibration if relevant)

# Discussion

What worked why

Where each model fails

Limitations

Practical operational implications

# Conclusion and future work

Direct answers to research

Summary of contributions

Next steps features ,odels and or deployment ect

# References

# Appendices